Vintage Computer Identification Data

Common text on labels, cases, PCB silkscreens, ICs, ROMs, assemblies, product codes, and serial labels

| Model candidate | Common observed text / identifier | Identifier type | Common variants / OCR equivalents | Identification context |
| --- | --- | --- | --- | --- |
| Commodore 64 / 64C | COMMODORE 64 | product text | C64; CBM 64; Commodore 64 Computer | case badge, label, manual, PCB association |
| Commodore 64 / 64C | COMMODORE 64C | product text | C64C; Commodore 64 C; 64C | case badge, late wedge case, cost-reduced board |
| Commodore 64 / 64C | SX-64 | product text | Executive 64; Commodore SX-64 | portable Commodore 64 family |
| Commodore 64 / 64C | PCB ASSY NO. 250469 | PCB assembly number | ASSY 250469; 250469 REV.A; 250469 REV.B; 250469 REV.E | strong Commodore 64C / late C64 board signal |
| Commodore 64 / 64C | Schematic NO. 252311 | schematic number | PCB NO. 252311; 252311 REV.A; 252311 REV.B | commonly paired with 250469 boards |
| Commodore 64 / 64C | 252312 | schematic / part number | Schematic 252312; 252312-left; 252312-right | 250469 Rev A schematic reference |
| Commodore 64 / 64C | PCB Assembly Number 250407-04 | PCB assembly number | ASSY 250407; 250407-01; 250407-04 Rev A; 250407 Rev A | breadbin C64 motherboard family |
| Commodore 64 / 64C | PCB Assembly Number 250425-01 | PCB assembly number | ASSY 250425; 250425 Rev B; 250425-01 Rev B | Commodore 64 motherboard family |
| Commodore 64 / 64C | 240441-01 | PCB assembly number | 240441; 240441-01 Rev B-2 | Commodore 64 motherboard variant |
| Commodore 64 / 64C | PCB Assembly Number 250466 | PCB assembly number | ASSY 250466; 250466 Rev B-3; 250466 | intermediate cost-reduced C64 board |
| Commodore 64 / 64C | PCB Assembly 326298-01 | PCB assembly number | ASSY 326298; 326298 Rev A; 326298-01 | early Commodore 64 motherboard family |
| Commodore 64 / 64C | KU-14194HB | PCB board identifier | KU14194HB; KU-14194 HB | European Commodore 64 board identifier |
| Commodore 64 / 64C | 326106 | schematic number | Schematic 326106 | schematic associated with 326298 board family |
| Commodore 64 / 64C | 251138 | schematic number | Schematic 251138 | schematic associated with 250407 board family |
| Commodore 64 / 64C | 251469 | schematic number | Schematic 251469 | schematic associated with 250425 / 240441 board family |
| Commodore 64 / 64C | 252278 | schematic number | Schematic 252278 | schematic associated with 250466 board family |
| Commodore 64 / 64C | MOS 6510 | chip marking | 6510; 8500; 8502 | C64-family CPU clue |
| Commodore 64 / 64C | MOS 6581 | chip marking | SID 6581; 6581R3; 6581R4 | C64 SID sound chip clue |
| Commodore 64 / 64C | MOS 8580 | chip marking | SID 8580; 8580R5 | C64C / later SID sound chip clue |
| Commodore 64 / 64C | MOS 6567 | chip marking | VIC-II 6567; 6567R56A | NTSC VIC-II video chip clue |
| Commodore 64 / 64C | MOS 6569 | chip marking | VIC-II 6569; 6569R3; 6569R5 | PAL VIC-II video chip clue |
| Commodore 64 / 64C | MOS 6526 | chip marking | CIA 6526; 6526A | C64 complex interface adapter clue |
| Commodore 64 / 64C | 901226-01 | ROM part marking | BASIC ROM 901226; MOS 901226-01 | C64 BASIC ROM clue |
| Commodore 64 / 64C | 901227-02 | ROM part marking | KERNAL ROM 901227; MOS 901227-02 | C64 KERNAL ROM clue |
| Commodore 64 / 64C | 901225-01 | ROM part marking | Character ROM 901225; MOS 901225-01 | C64 character ROM clue |
| Commodore 64 / 64C | silver label | visual label text | silver metal label; silver sticker; rainbow badge | external C64 case clue |
| Commodore 64 / 64C | SERIAL NO. | serial label text | SER.NO.; S/N; SN; serial | C64 external label; combine with model and board evidence |
| Apple II family | Apple ][ | product text | Apple II; Apple II Computer; apple ][ | original Apple II family cue |
| Apple II family | Apple II Plus | product text | Apple ][ Plus; Apple II+; II Plus | Apple II Plus model cue |
| Apple II family | Apple IIe | product text | Apple //e; Apple IIe Enhanced; Platinum IIe | Apple IIe model cue |
| Apple II family | A2S1 | model / serial prefix | A2S1-0101; A2S1-14934; A2S1-27425; A2S1 serial | Apple II serial/model prefix cue |
| Apple II family | A2S10 | model / serial prefix | A2S10; A2S10-xxxxx; Apple II Plus A2S10 | Apple II Plus serial/model prefix cue |
| Apple II family | A2S2064 | model / serial prefix | Apple IIe A2S2064; A2S2064 label | Apple IIe label cue |
| Apple II family | A2S2128 | model / serial prefix | Apple IIe A2S2128; A2S2128 label | Apple IIe / 128K label cue |
| Apple II family | A2S6000 | model / serial prefix | Apple IIGS A2S6000; IIGS A2S6000 | Apple IIGS model cue |
| Apple II family | A2S6010 | model / serial prefix | A2S6010; Apple IIGS A2S6010 | Apple IIGS model cue |
| Apple II family | 820-0001 | logic board part family | 820-0001-01; 820-0001-03; 820-0001-04; 820-0001-07 | Apple II motherboard / logic board identifier family |
| Apple II family | 820-0001-03 | logic board part number | Rev 3; Revision 3; 820-0001-03 | Apple II Rev 3 board cue |
| Apple II family | 820-0001-04 | logic board part number | Rev 4; Revision 4; 820-0001-04 | Apple II Rev 4 board cue |
| Apple II family | REV 0 | board revision | Rev.0; Revision 0; rev0 | very early Apple II board cue |
| Apple II family | REV 3 | board revision | Rev.3; Revision 3; REV3 | Apple II board revision cue |
| Apple II family | REV 4 | board revision | Rev.4; Revision 4; REV4 | Apple II board revision cue |
| Apple II family | Integer BASIC ROM | ROM / firmware text | INTEGER BASIC; Integer ROM; non-Autostart ROM | early Apple II firmware cue |
| Apple II family | Autostart ROM | ROM / firmware text | AUTOSTART; Autostart Monitor ROM | Apple II / II Plus firmware cue |
| Apple II family | Datanetics | keyboard marker | Datanetics K-160; K-160 | early Apple II keyboard clue |
| Apple II family | Disk II | peripheral/product text | Disk ][; Apple Disk II; A2M0003 | Apple II floppy system cue |
| Apple II family | 6502 | chip marking | MOS 6502; Synertek 6502; Rockwell 6502 | Apple II CPU clue |
| MITS Altair 8800 | ALTAIR 8800 | product text | Altair 8800; MITS Altair 8800 | front panel / manual / label cue |
| MITS Altair 8800 | MITS | manufacturer text | MITS, Inc.; Micro Instrumentation and Telemetry Systems | Altair manufacturer cue |
| MITS Altair 8800 | Creative Electronics | label text | MITS Creative Electronics; Creative Electronics Albuquerque | rear serial label cue |
| MITS Altair 8800 | 8800 DISPLAY/CONTROL BOARD | board text | Display Control Board; Display/Control Board Assembly | Altair front panel board cue |
| MITS Altair 8800 | 8800 CPU BOARD | board text | CPU Board Assembly; Altair CPU Board | Altair 8080 processor board cue |
| MITS Altair 8800 | 1K STATIC MEMORY BOARD | board text | Static Memory Board; 1K RAM Board | Altair memory board cue |
| MITS Altair 8800 | 8800 POWER SUPPLY BOARD | board text | Power Supply Board Assembly | Altair power board cue |
| MITS Altair 8800 | EXPANDER BOARD | board text | Expander Board 8800 M/Bd; 8800 motherboard | Altair bus/motherboard cue |
| MITS Altair 8800 | S-100 | bus text | S100; S-100 bus; Altair bus | Altair / early microcomputer bus cue |
| MITS Altair 8800 | Intel 8080 | chip marking | 8080A; C8080A; 8080 CPU | Altair CPU clue |
| MITS Altair 8800 | 220128 | serial example | serial 220128 | known observed Altair-style serial example; do not infer chronology |
| MITS Altair 8800 | 220765K | serial example | serial 220765K | known observed Altair-style serial example; do not infer chronology |
| MITS Altair 8800 | 222514K | serial example | serial 222514K | known observed Altair-style serial example; do not infer chronology |
| MITS Altair 8800 | SERIAL NO. | serial label text | S/N; SN; Serial Number; MITS Serial | Altair rear label text; serial sequence alone is weak evidence |
| Sinclair ZX Spectrum family | ZX Spectrum | product text | Sinclair ZX Spectrum; Spectrum 16K; Spectrum 48K | original rubber-key Spectrum cue |
| Sinclair ZX Spectrum family | ZX Spectrum+ | product text | Spectrum Plus; ZX Spectrum Plus; ZX Spectrum + | later keyboard/case Spectrum cue |
| Sinclair ZX Spectrum family | ZX Spectrum 128 | product text | Spectrum 128K; ZX Spectrum 128K; Toastrack | 128K Spectrum cue |
| Sinclair ZX Spectrum family | ZX Spectrum +2 | product text | Spectrum +2; Grey +2; +2 grey | Amstrad grey +2 cue |
| Sinclair ZX Spectrum family | ZX Spectrum +2A | product text | Spectrum +2A; black +2A; +2A | black +2A cue |
| Sinclair ZX Spectrum family | ZX Spectrum +2B | product text | Spectrum +2B; black +2B; +2B | black +2B cue |
| Sinclair ZX Spectrum family | ZX Spectrum +3 | product text | Spectrum +3; +3; disk drive | integrated 3 inch floppy Spectrum cue |
| Sinclair ZX Spectrum family | ISSUE 1 | PCB issue marking | Issue One; ISSUE ONE; Issue 1 PCB | early Spectrum 16/48K board cue |
| Sinclair ZX Spectrum family | ISSUE 2 | PCB issue marking | Issue Two; ISSUE TWO; Issue 2 PCB | Spectrum 16/48K board cue |
| Sinclair ZX Spectrum family | ISSUE 3 | PCB issue marking | Issue Three; ISSUE THREE; Issue 3 PCB | Spectrum 16/48K board cue |
| Sinclair ZX Spectrum family | ISSUE 3B | PCB issue marking | Issue 3B; ISSUE 3B | Spectrum 16/48K board cue |
| Sinclair ZX Spectrum family | ISSUE 4A | PCB issue marking | Issue 4A; ISSUE 4A | Spectrum 16/48K board cue |
| Sinclair ZX Spectrum family | ISSUE 4B | PCB issue marking | Issue 4B; ISSUE 4B | Spectrum 16/48K board cue |
| Sinclair ZX Spectrum family | ISSUE 5 | PCB issue marking | Issue Five; ISSUE FIVE; Issue 5 PCB | Spectrum 16/48K board cue |
| Sinclair ZX Spectrum family | ISSUE 6A | PCB issue marking | Issue 6A; ISSUE 6A | Spectrum 16/48K board cue |
| Sinclair ZX Spectrum family | Z70500 | PCB board code | Z70500 Issue 3; +2 Z70500 | ZX Spectrum +2 family motherboard cue |
| Sinclair ZX Spectrum family | Z70830 | PCB board code | Z70830 ISSUE 1; Z70830 ISSUE 2 | ZX Spectrum +2A / +3 common motherboard cue |
| Sinclair ZX Spectrum family | Z70833 | PCB board code | Z70833 ISSUE 1; Z70833 ISSUE 2 | ZX Spectrum +2B motherboard cue |
| Sinclair ZX Spectrum family | Z70835 | PCB board code | Z70835 ISSUE 1 | ZX Spectrum +3B motherboard cue |
| Sinclair ZX Spectrum family | D serial prefix | serial prefix | Dxxxxx; D serial; Dundee prefix | reported Spectrum serial/manufacture cue; use with other evidence |
| Sinclair ZX Spectrum family | 001 serial prefix | serial prefix | 001xxxxx; 001 prefix | reported Spectrum serial/manufacture cue; use with other evidence |
| Sinclair ZX Spectrum family | ULA | chip marking | 5C102E; 5C112E; 6C001E; Amstrad 40077 | Spectrum custom logic clue |
| Sinclair ZX Spectrum family | Z80A | chip marking | Zilog Z80A; Z8400; NEC D780C | Spectrum CPU clue |
| Sinclair ZX Spectrum family | AY-3-8912 | chip marking | AY38912; Yamaha YM2149 | 128K Spectrum sound chip clue |
| Sinclair ZX Spectrum family | rubber keys | visual clue | rubber keymat; blue keymat; grey keymat | original Spectrum external cue |
| General hardware OCR cues | REV | generic revision text | REV.A; REV.B; REV C; Revision; Rev. | capture with nearby board number |
| General hardware OCR cues | ISSUE | generic revision text | ISSUE 1; ISSUE TWO; Issue 3B | especially useful for ZX Spectrum boards |
| General hardware OCR cues | ASSY | generic assembly text | PCB ASSY; ASSY NO.; Assembly No.; assembly number | often near board identifiers |
| General hardware OCR cues | P/N | generic part text | PN; Part No.; Part Number; P/N: | often near hardware identifiers |
| General hardware OCR cues | SERIAL | generic serial text | Serial No.; S/N; SN; Ser. No.; Serial Number | capture full neighboring string |
| General hardware OCR cues | MADE IN | generic label text | Made in USA; Made in England; Made in Hong Kong; Made in Japan; Made in Scotland | manufacturing context clue |
| General hardware OCR cues | COPYRIGHT | generic PCB text | ©; (C); Copyright; 1982; 1983; 1987 | capture year and nearby board text |
| General hardware OCR cues | YYWW chip date code | date-code pattern | four digits such as 8234, 8312, 8705 | chip date clue only; not a serial number |